https://www.tinkercad.com/things/7BZEyVGHnRF-copy-of-grand-uusam?sharecode=Islu1q9mtl8IX7uWKDZU_EkkUHbedGXtKmVug6u7Fa8

Master code

#include <Wire.h>

#include <Servo.h>

const int LDR_PIN = A0;

const int GAS_PIN = A1;

const int TEMP_PIN = A2;

const int SERVO_PIN = 9;

const int BUZZ_PIN = 8;

const byte MASTER_ADDRESS = 8;

const int GAS_ALERT_THRESHOLD = 180;

const int GAS_CLEAR_THRESHOLD = 130;

const float TEMP_EMERGENCY_THRESHOLD = 45.0;

const int BLACKOUT_DROP = 100;

const int LIGHT_RESTORE_THRESHOLD = 150;

const byte CMD_ACTIVATE = 0;

const byte CMD_RESET = 16;

enum SystemState {

STANDBY,

ACTIVE,

GAS_ALERT,

BLACKOUT_ALERT,

TEMPERATURE_EMERGENCY,

MULTI_FAULT

};

SystemState currentState = STANDBY;

Servo ventServo;

int ldrValue = 0;

int gasValue = 0;

float temperature = 0.0;

bool gasAlertActive = false;

bool blackoutActive = false;

int previousLdrValue = 0;

void requestEvent() {

byte tempByte = (byte)constrain(

(int)temperature,

0,

255

);

byte lightByte = map(

constrain(ldrValue, 0, 1023),

0,

1023,

0,

255

);

byte gasByte = map(

constrain(gasValue, 0, 1023),

0,

1023,

0,

255

);

Wire.write(tempByte);

Wire.write(lightByte);

Wire.write(gasByte);

Wire.write((byte)currentState);

}

void receiveEvent(int howMany) {

if (howMany <= 0) {

return;

}

byte command = Wire.read();

while (Wire.available()) {

Wire.read();

}

if (command == CMD_ACTIVATE) {

if (currentState == STANDBY) {

currentState = ACTIVE;

previousLdrValue = ldrValue;

gasAlertActive = false;

blackoutActive = false;

Serial.println("SYSTEM ACTIVATED");

Serial.print("INITIAL LDR REFERENCE: ");

Serial.println(previousLdrValue);

}

}

else if (command == CMD_RESET) {

if (currentState == TEMPERATURE_EMERGENCY) {

if (temperature <= TEMP_EMERGENCY_THRESHOLD) {

ventServo.write(0);

currentState = ACTIVE;

previousLdrValue = ldrValue;

Serial.println("TEMPERATURE EMERGENCY RESET");

Serial.println("SERVO -> 0 DEGREES");

}

else {

Serial.println(

"RESET DENIED - TEMPERATURE STILL HIGH"

);

}

}

}

}

void readTemperature() {

int rawTemp = analogRead(TEMP_PIN);

float voltage = rawTemp * (5.0 / 1023.0);

temperature = (voltage - 0.5) * 100.0;

}

void readSensors() {

if (currentState == STANDBY) {

return;

}

ldrValue = analogRead(LDR_PIN);

gasValue = analogRead(GAS_PIN);

readTemperature();

}

void updateGasFault() {

if (!gasAlertActive) {

if (gasValue > GAS_ALERT_THRESHOLD) {

gasAlertActive = true;

Serial.println("GAS ALERT DETECTED");

}

}

else {

if (gasValue < GAS_CLEAR_THRESHOLD) {

gasAlertActive = false;

Serial.println("GAS LEVEL SAFE");

}

}

}

void updateBlackoutFault() {

if (blackoutActive) {

if (ldrValue >= LIGHT_RESTORE_THRESHOLD) {

blackoutActive = false;

Serial.println("LIGHT RESTORED");

}

return;

}

int lightDecrease =

previousLdrValue - ldrValue;

if (lightDecrease >= BLACKOUT_DROP) {

blackoutActive = true;

Serial.println("BLACKOUT DETECTED");

Serial.print("Previous LDR: ");

Serial.println(previousLdrValue);

Serial.print("Current LDR: ");

Serial.println(ldrValue);

Serial.print("Light decrease: ");

Serial.println(lightDecrease);

}

}

void updateStateMachine() {

if (currentState == STANDBY) {

digitalWrite(BUZZ_PIN, LOW);

return;

}

if (temperature > TEMP_EMERGENCY_THRESHOLD) {

if (currentState != TEMPERATURE_EMERGENCY) {

currentState = TEMPERATURE_EMERGENCY;

ventServo.write(180);

Serial.println("======================");

Serial.println("TEMPERATURE EMERGENCY!");

Serial.println("SERVO -> 180 DEGREES");

Serial.println("======================");

}

return;

}

if (currentState == TEMPERATURE_EMERGENCY) {

ventServo.write(180);

return;

}

updateGasFault();

updateBlackoutFault();

if (gasAlertActive && blackoutActive) {

currentState = MULTI_FAULT;

digitalWrite(BUZZ_PIN, HIGH);

return;

}

if (currentState == MULTI_FAULT) {

if (gasAlertActive && blackoutActive) {

digitalWrite(BUZZ_PIN, HIGH);

return;

}

if (gasAlertActive && !blackoutActive) {

currentState = GAS_ALERT;

digitalWrite(BUZZ_PIN, LOW);

return;

}

if (!gasAlertActive && blackoutActive) {

currentState = BLACKOUT_ALERT;

digitalWrite(BUZZ_PIN, LOW);

return;

}

if (!gasAlertActive && !blackoutActive) {

currentState = ACTIVE;

digitalWrite(BUZZ_PIN, LOW);

return;

}

}

if (currentState == GAS_ALERT) {

if (gasAlertActive && blackoutActive) {

currentState = MULTI_FAULT;

digitalWrite(BUZZ_PIN, HIGH);

return;

}

if (gasAlertActive) {

digitalWrite(BUZZ_PIN, LOW);

return;

}

if (!gasAlertActive) {

if (blackoutActive) {

currentState = BLACKOUT_ALERT;

}

else {

currentState = ACTIVE;

}

digitalWrite(BUZZ_PIN, LOW);

return;

}

}

if (currentState == BLACKOUT_ALERT) {

if (gasAlertActive && blackoutActive) {

currentState = MULTI_FAULT;

digitalWrite(BUZZ_PIN, HIGH);

return;

}

if (blackoutActive) {

digitalWrite(BUZZ_PIN, LOW);

return;

}

if (!blackoutActive) {

if (gasAlertActive) {

currentState = GAS_ALERT;

}

else {

currentState = ACTIVE;

}

digitalWrite(BUZZ_PIN, LOW);

return;

}

}

if (currentState == ACTIVE) {

digitalWrite(BUZZ_PIN, LOW);

if (gasAlertActive && blackoutActive) {

currentState = MULTI_FAULT;

digitalWrite(BUZZ_PIN, HIGH);

}

else if (gasAlertActive) {

currentState = GAS_ALERT;

}

else if (blackoutActive) {

currentState = BLACKOUT_ALERT;

}

}

}

void setup() {

Serial.begin(9600);

pinMode(BUZZ_PIN, OUTPUT);

digitalWrite(BUZZ_PIN, LOW);

ventServo.attach(SERVO_PIN);

ventServo.write(0);

Wire.begin(MASTER_ADDRESS);

Wire.onRequest(requestEvent);

Wire.onReceive(receiveEvent);

ldrValue = 0;

gasValue = 0;

temperature = 0.0;

previousLdrValue = 0;

currentState = STANDBY;

Serial.println("======================");

Serial.println("MASTER READY");

Serial.println("STATE: STANDBY");

Serial.println("SENSORS INACTIVE");

Serial.println("======================");

}

void loop() {

readSensors();

updateStateMachine();

Serial.print("Temp: ");

Serial.print(temperature);

Serial.print(" C | LDR: ");

Serial.print(ldrValue);

Serial.print(" | Gas: ");

Serial.print(gasValue);

Serial.print(" | State: ");

Serial.println(currentState);

if (currentState != STANDBY) {

previousLdrValue = ldrValue;

}

delay(200);

}

Slave Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <IRremote.h>

LiquidCrystal_I2C lcd(0x20, 16, 2);

const int IR_PIN = 11;

const byte POWER_BUTTON = 0;

const byte ZERO_BUTTON = 12;

const byte ONE_BUTTON = 16;

const byte SENSOR_ARDUINO = 8;

const byte STANDBY = 0;

const byte ACTIVE = 1;

const byte GAS_ALERT = 2;

const byte BLACKOUT_ALERT = 3;

const byte TEMPERATURE_EMERGENCY = 4;

const byte MULTI_FAULT = 5;

byte temperature = 0;

int lightValue = 0;

int gasValue = 0;

byte currentState = STANDBY;

bool showLight = true;

void sendCommand(byte command) {

Wire.beginTransmission(SENSOR_ARDUINO);

Wire.write(command);

Wire.endTransmission();

}

bool getSensorData() {

Wire.requestFrom(SENSOR_ARDUINO, 4);

if (Wire.available() >= 4) {

temperature = Wire.read();

byte light = Wire.read();

lightValue = map(light, 0, 255, 0, 1023);

byte gas = Wire.read();

gasValue = map(gas, 0, 255, 0, 1023);

currentState = Wire.read();

return true;

}

return false;

}

void displayStandby() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("AWAITING RITUAL");

}

void displayActive() {

lcd.clear();

if (showLight) {

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(temperature);

lcd.print("C L:");

lcd.print(lightValue);

lcd.setCursor(0, 1);

lcd.print("G:");

lcd.print(gasValue);

}

else {

int airPurity = map(

constrain(gasValue, 0, 1023),

0, 1023,

100, 0

);

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(temperature);

lcd.print("C A:");

lcd.print(airPurity);

lcd.print("%");

lcd.setCursor(0, 1);

lcd.print("G:");

lcd.print(gasValue);

}

}

void displayGasAlert() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("TOXIC PURGE");

}

void displayBlackout() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("NOCTIS");

lcd.setCursor(0, 1);

lcd.print("PROTOCOL");

}

void displayTemperatureEmergency() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("COOKED");

}

void displayMultiFault() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("MULTIPLE");

lcd.setCursor(0, 1);

lcd.print("PROBLEMS DETECT");

}

void handleIR() {

if (!IrReceiver.decode()) {

return;

}

byte command = IrReceiver.decodedIRData.command;

Serial.print("IR Command: ");

Serial.println(command);

if (command == POWER_BUTTON) {

if (currentState == STANDBY) {

sendCommand(POWER_BUTTON);

Serial.println("ACTIVATION SENT");

delay(200);

}

}

else if (command == ZERO_BUTTON) {

if (currentState == ACTIVE) {

showLight = !showLight;

Serial.println("DISPLAY TOGGLED");

}

}

else if (command == ONE_BUTTON) {

if (currentState == TEMPERATURE_EMERGENCY) {

sendCommand(ONE_BUTTON);

Serial.println("RESET SENT");

delay(200);

}

}

IrReceiver.resume();

}

void setup() {

Serial.begin(9600);

Wire.begin();

lcd.init();

lcd.backlight();

IrReceiver.begin(

IR_PIN,

ENABLE_LED_FEEDBACK

);

currentState = STANDBY;

displayStandby();

Serial.println("---------------------");

Serial.println("SLAVE READY");

Serial.println("STATE: STANDBY");

Serial.println("=---------------------");

}

void loop() {

handleIR();

getSensorData();

switch (currentState) {

case STANDBY:

displayStandby();

break;

case ACTIVE:

displayActive();

break;

case GAS_ALERT:

displayGasAlert();

break;

case BLACKOUT_ALERT:

displayBlackout();

break;

case TEMPERATURE_EMERGENCY:

displayTemperatureEmergency();

break;

case MULTI_FAULT:

displayMultiFault();

break;

}

delay(300);

}